TECHNICAL OFFER FORM

Open call for tender for the supply of audio/visual equipment including installation, configuration and the provision of support services for Sala Europa of Villa Schifanoia and C13, D1 and D2 seminar rooms of Palazzo Buontalenti

Ref: OP/EUI/ICTS/2024/003

The undersigned ________________________________tax identification n. _________________

born in _________________________________________on_____________________________

address _______________________________________________________________________

email address _________________________ PEC (certified) email address _________________

- representing the following legal person:

⇨ full official name:

⇨ official legal form:

⇨ full official address:

⇨ VAT registration number:

SUBMITS THE FOLLOWING TECHNICAL OFFER:

| TECHNICAL AND QUALITY ASPECTS TO BE EVALUATED The technical score (max of 3 points) will be assigned based on the technical sheets of the following equipment: |  |  |  |
| --- | --- | --- | --- |
| AUDIO VISUAL EQUIPMENT |  |  |  |
| DESCRIPTION |  | Model offered | Max score |
| A1 | Indoor Led wall, Dimension 3600x2022 mm COB Cabinet equipped with flip-chip and receiving card, which rounds off the image with various software functions. Key features: Pixel Pitch 0.9 0.9375 mm pixel pitch 1,200 nit 2 A10s Pro Receiving Cards/ per cabinet Processing COB (Chip on Board) flip-chip technology Viewing angle: 160° horizontal:160° vertical Refreshrate: 7.680 Hz Bit depth: 16 bit Pixel Density: 640x360 px/cabinet [Reference Model: LANG LEDgend 0.9] | _____________________ | 3 points |
| A2 | IP 4K Pan Tilt Zoom camera with PTZ Auto Framing and NDI ¦ HX capability (via optional license). The sensitive 1/2.5-type Exmor R sensor shall capture high-quality 4K images with very low noise, even in dimly lit environments. High quality 20x optical zoom and 30x (4K) or 40x (FHD) with Clear Image Zoom: The high-quality lens with 20x optical zoom range shall cover wide angle shots as well as tight close-ups. Clear Image Zoom shall expand this range to 30x (4K) or 40x (FHD) without sacrificing detail. In addition, the Tele Convert Mode shall be able to double this range again, up to 80x zoom while maintaining a 1920 x 1080 resolution. [Reference Model: SONY SRG-A40 or equivalent] | _____________________ | 3 points |
| A3 | Monitor Touch 75” Anti-Gloss treatment, Anti-Glare glass, Matt polished surface, Ultra smooth writing, Zero Air-Gap, 3840 x 2160 (8.3 megapixel 4K UHD), Static Contrast 5000:1, horizontal/vertical: 178°/178°, right/left: 89°/89°, up/down: 89°/89°, PureTouch-IR⁺, Touch Methods: stylus, finger, glove (Passive pen, Opaque objects), Digital Signal Input: HDMI x4 (2.0, max. 3840x2160 @60Hz, YUV420/ YUV444/ RGB444) DisplayPort x1 (2.1, max. 3840x2160 @30Hz) USB-C x2 (v.3.2 (Gen 1, 5Gbit) 3840x2160 @60Hz, RGB444 ¦ DP 1.2 Alt mode, Touch, 1x front: 100W PD , 1x back: 15W PD), Digital Signal Output: HDMI x1 (2.0, max. 3840x2160 @60Hz, MSD (Multi-Screen Display)) USB-C x1 (v.3.2 (Gen 1, 5Gbit), 3840x2160 @60Hz (DP 1.2 Alt mode, Touch, 5W PD, MSD (Multi-Screen Display)), Integrated Software: iiWare 11 (Android 13 OS) featuring iiyama DMS, Note, web browser, file manager, cloud drives, WPS office, iiyama Share and EShare for wireless connection with Windows/iOS/Android devices. [Reference Model: iiYama 75TE7514MIS-B1AG] | _____________________ | 3 points |
| A4 | Audio digital processor provides a fully integrated audio, video, and control solution for installations with a mixture of analog and network channels, supporting medium-sized rooms through the largest Enterprise scale deployments. Device Height: 44 mm (1.75 in) Device Width: 483 mm (19 in) Device Depth: 282.5 mm (11.12 in) Device Weight: 5 kg (11 lb) Shipping Height: 152 mm (6 in) Shipping Width: 584 mm (23 in) Shipping Depth: 356 mm (14 in) Shipping Weight: 5.6 kg (12.4 lb) Key Features: 128 networked audio channels (AES67) 24 channels of analog I/O, including 8 configurable Flex channels 16x AEC processors Supports up to 4x network microphones Up to 32 x 32 Dante audio channels (8 x 8 included) USB AV bridging (16 x 16 audio) External USB audio device host Up to 4x VoIP instances [Reference model Q-Sys a110f or equivalent] | _____________________ | 3 points |
| A5 | Compact 16x2 inch Passive Line Column Array Speakers, Coverage V.7°I H.90° Frequency Response: 135 Hz – 20kHz (-6dB) Max SPL: 123dB Rated Power: 400W Nominal Impedance: 8 – 32 Ohm Dimensions: 60x1204x66mm Weight: 3,2 kg. [Reference Model: K-Gear GF162I] | _____________________ | 3 points |
| A6 | Wireless System with Body Pack Transmitter and Lavalier Microphone Frequency Range 470-937.5 MHz Operate up to 32 compatible systems per 44 MHz band, with easy setup and optional rechargeability accessories for wireless excellence, wherever required. [Reference model: Shure SLXD14/85 or equivalent] | _____________________ | 3 points |
| A7 | Wireless System with Cardioid Dynamic Microphone Frequency Range 470-937.5 MHz Operate up to 32 compatible systems per 44 MHz band, with easy setup and optional rechargeability accessories for wireless excellence, wherever required. [Reference Model: Shure SLXD24/SM58 or equivalent] | _____________________ | 3 points |
| A8 | Ceiling Array Microphone Automatic Coverage™ AES67 and Dante Digital Output 10 total channels 8 (independent) transmit channels, 1 automix output, 1 AEC reference in channel Sampling Rate: 48kHz Bit Depth: 24 Acoustic Echo Cancellation: Up to 250ms Frequency response: 125-20.000 kHz [Reference Model: SHURE MXA920] | _____________________ | 3 points |
| A9 | Wireless Presenter which enables secure wireless collaboration in the modern digital workspace. 100/1000 Mbps, auto-switching, autonegotiating, autodiscovery, full/half duplex, TCP/IP, UDP/IP, DHCP, SSL, TLS5, SSH, SFTP (SSH File Transfer Protocol), IEEE 802.1x, Active Directory authentication, HTTPS web browser setup and XiO Cloud service, 802.3af compliant USB 2.0 for connecting a USB conferencing peripheral; USB 3.0 for connecting a USB conferencing peripheral. HDMI Input: 1920x1080@60Hz (HD 1080p60); AirMedia presentation: 1920x1080@30Hz (1080p30) Up to 30 fps, audio supported. [Reference Model: Crestron AM-3200] | _____________________ | 3 points |

| CONTROL SYSTEMS EQUIPMENT (SALA EUROPA) |  |  |  |
| --- | --- | --- | --- |
| DESCRIPTION |  | Model offered | Max score |
| A10 | Provision of processor, 4-series controller, 2 GB SDRAM, 8 GB flash memory, Embedded 4-Series multicore CPU processor, IPv6 ready. Dual Network Port: (1) Ethernet (2) Control Subnet [Reference Model: CRESTRON CP4N or equivalent] | _____________________ | 3 points |

The tenderer is required to submit a technical offer duly filling in the above table, point by point.

Date: __/__/2024

Signature of the economic operator’s legal representative:

______________________________________________